HISTORY

| DOCUMENT OWNER | DOC REV# | DESCRIPTION OF CHANGE | DATE |
| --- | --- | --- | --- |
| McMillan, Zachary | 000 | Initial Release | {date} |

# PURPOSE:

The purpose of this procedure is to define responsibility, authority and steps for testing {Author} PCA’s per customer specifications.

# SCOPE:

This procedure applies to all personnel performing testing.

# REFERENCE DOCUMENTS:

## ISO 9001 / AS9100 / ISO 13485 Standards: Current Revisions

## Quality Manual #422

## Document and Data Control Procedure #423

## Control of Records Procedure #424

## PCA Test Procedure P0164

# RESPONSIBILITIES:

## The technician is responsible for performing tests on applicable PC Assemblies (PCA’s);

## The technician has authority to reject and trouble shoot defective PCA’s/components;

## The technician is responsible for completing a Product Test Record for each test performed.

# EQUIPMENT:

## ESD compliant workspace

## Figure 1

## Figure 2

# PROCEDURE:

(SETUP)

## Document No.: Rev:
Programming Procedure
37056 2
PROGRAMMING PROCEDURE, HALO
DEVICE Page 1 of 2
Introduction:
This document describes the steps needed to load firmware into the Halo device.
1. Programming Setup:
1.1. Ensure the USB flash drive fixture (Item 921) is installed in the Halo Device
Programming Fixture Assembly (Item 909) and the computer is powered on.
Computer may be plugged into AC outlet as needed for battery capacity.
1.2. Open the folder on the flash drive and verify the software revision numbers,
which will be displayed as part of the zip file name (Example:
“sem_app_dfu_V_0_4_6.zip”), match the revision number of the controlled-copy
software specs (Item 907 and 912).
1.3. If the software revision number does not match, remove all files and replace with
the current revision by following the Assembly Procedure for the USB flash
drive fixture (Item 921).
1.4. Save a copy of the .zip files on the USB drive to the desktop.
1.5. Record the assembly serial number and software versions/builds (all the
numbers at the end of the hex file’s name, example: for
sem_app_dfu_V_0_4_6.zip record 0.4.6) on the Test Record (Item 914)
1.6. Record the Halo PCB serial number on the Test Record (Item 914).
2. Initial Hardware Setup:
2.1. Hold the button down on the Halo device for 8 seconds until the device LED
shines blue.
2.2. Connect USB micro cable (Item 905) between the Halo device and the
programming computer (Item 909).
2.3. Using the Device Manager (Windows Key ->Device Manager), find the COM
port the Halo device is connected to (it will be called nRF52).
3. Programming Procedure:
3.1. Open a command prompt from the start menu.
3.2. Enter the command “nrfutil dfu usb-serial -pkg <zip-file-name> -p <device-
COM-port>” substituting the dfu file name of the software being loaded and
the COM port determined in step 2.3
3.3. If this was the final device to be programmed, close the command prompt.
© Sempulse 2020-24. The information contained in this document is the sole property of Sempulse Corporation. Any
reproduction or use in part or whole without the written permission of Sempulse is prohibited.

--- Page 2 ---
Document No.: Rev:
Programming Procedure
37056 2
PROGRAMMING PROCEDURE, HALO
DEVICE Page 2 of 2
3.4. Press the button on the front of the device for 3 seconds to power off the
device.
3.5. Press the button on the front of the board for 3 seconds to power on the
device.
3.6. Verify that the LED is blinking yellow and red if loading the application
software or solid white for the test software.
3.7. Press the button on the front of the device for 3 seconds to power off the
device.
3.8. Disconnect the USB micro to USB cable from the Halo device.
3.9. Once programming is completed, delete the software file from the desktop.
4. Have a second individual verify the software version loaded matches the software
version on the released spec and complete the Test Record (Item 914).
Revision History
Rev Date Change No. Originator Description of Change
1 10-06-20 P-SEMP-008 D. Miller Initial release.
Updated to use dfu with nrfutil so updates are cryptographically signed. Update to
2 11-06-20 P-SEMP-022 D. Miller
use USB drive and computer as separate fixtures rather than fixture assembly.
© Sempulse 2020-24. The information contained in this document is the sole property of Sempulse Corporation. Any
reproduction or use in part or whole without the written permission of Sempulse is prohibited.

(REPEAT)

## FAILED BOARDS:

## Failed UUTs are to be retested, and if UUT fails a second time, technician will troubleshoot for failures and rework as applicable.

## All reworked UUTs are to be retested.

# Relative photos

Figure 1

Figure 2

Figure 3

Figure 4

Figure 5