1.	We commonly have issues where the vendor doesn�t set the fuses correctly. And the divide by 8 is left on. This causes the max speed to be 50 Cycles per min.
so if you can verify that is correct then that would be extremely helpful. Then after it is set then turn it back to the off position 0 VDC.
2.	Are we tuning anything with the trim pot TM1?
a.	Right now we have not been calibrating anything to the trim pot. We are simply using VCC as max and GND as low.
b.	The trim pot can be used for testing for verification of the program.
i.	When set to 0 vdc the output shuld have 0 cicles per min
ii.	When set to 5 VDC the output shuld be 400 cicls per min.
3.	Is there a blinking LED after the assembly is programmed that needs to be verified to be sure it was programed currectly?
a.	The customer requested that we remove the test LEDs because they were distracting. The pads are styll there but are just not populated.
b.	I can send you a valve with an LED indicator that you can put on the device and view the sycle rate.
4.	fixturs needed
atmel ice programer/TF038
TF045 WHITE KNIGHT 8600-XX-0026 (all suplys in box)
5.       programe/ test instruction
a.  open microchip studio.
a1. go to (tools>device programing). set up like this ([atmel-ice] [ATtiny841 or ATtiny441] [isp]) pick coralted softwair in memories.
a2. go to fuses and set it to ([32 kHz] [disabled] [disabled] [on] [off] [off] [on] [off] [on] [n/a] [off] [off] [int. rc osc.8mHz;])
6.	set power suply to 24v. Atatch fixtur wiers from (TF045) to the powur supplie. Then plug in sulonoid and power in to the board.
6a. plug (TF038) in to computer and conect the riben wier with pogo pins to the avr slot on the programer. push pins against the corsponding pads on the back
of the board and use the hot key that is in the firmwair folder to program the board.
7.	sort T-pot/trimpot acrostpintuand3 the led should blink faster. Then sort 1and2 it should blinck slower. Turn t-pot to 0 sycles per min and marck corner with
gold sharpy to show it is done.