HISTORY

| DOCUMENT OWNER | DOC REV# | DESCRIPTION OF CHANGE | DATE |
| --- | --- | --- | --- |
| Fuller, Ethan | 002 | Loctite 403 in Equipment and glue procedure 6.13 | 1/23/2023 |
| Fuller, Ethan | 001 | Updated Logo & Reference Documents. Removed Sections 8 & 9. Included programmers in equipment. | 2/10/2022 |
| Brunson, Travis | 000 | Initial Release | 10/3/2016 |

# PURPOSE:

The purpose of this procedure is to define responsibility, authority and steps for testing Quantronix / Cubiscan 12841 Receiver PCA’s per customer specifications.

# SCOPE:

This procedure applies to all personnel performing testing.

# REFERENCE DOCUMENTS:

## ISO 9001 / AS9100 / ISO 13485 Standards: Current Revisions

## Quality Manual #422

## Document and Data Control Procedure #423

## Control of Records Procedure #424

## PCA Test Procedure P0164

# RESPONSIBILITIES:

## The technician is responsible for performing tests on applicable PC Assemblies (PCA’s);

## The technician has authority to reject and trouble shoot defective PCA’s/components;

## The technician is responsible for completing a Product Test Record for each test performed.

# EQUIPMENT:

## ESD compliant workspace

## Quantronix Cubiscan 12841 Receiver Test Fixture (TF033)

## USB Blaster (TF042)

## MPlab ICD 3 (TF042)

## Loctite 403

# PROCEDURE:

(SETUP)

## Locate the Quantronix Cubiscan 12841 test fixture (TF033).  There are two.  The one used to program the Rx has a 3 pin power plug while the TX has a 2 Pin.

### Connect 120 VAC Power cord to a Power Strip with an on/off switch.

### Connect the DB-9 Serial Cable through a serial/usb converter (USB) to a PC.

### Connect the USB Blaster Programming Module (USB) to a PC.

### Connect the MPlab ICD-3 Programming Puck (USB) (RX Only) to a PC.

### Run Quantronix software titled “IR Gate Config.exe” and select the tabs for Rx.

### If necessary, select the appropriate Rx programming files from the Quantronix Tx/Rx Firmware folder.

(Test)

## Install one jumper at P4 position 1 which is pins 1 & 2.

## Place the UIT onto the fixture using the two locator pins with the LED’s facing downward.

## Connect serial cable/cables to the UIT.

## Align and Connect the ICD3 to port P3

## Align and Connect the USB Blaster to port P5.

## Align and Connect the Power Plug to Port P6 or P7 (Doesn’t matter which one)

## Power up the UIT via the power strip.  Power LED should light.

## Using IR Gate Config; program the:

### Hit F3 to kick off Altera Micro-controller (USB Blaster) which takes about 1 second.  Watch for confirmation.  Press any key then close the window.

### Hit F4 to kick off Pic Micro-controller (ICD3) which takes a few seconds.  Watch for confirmation.

## Press any key, close the “Cycle Power” dialog and Cycle Power.

## Select the Test button and wait for the Green progress bar to finish.  (About 5 seconds).

## Any errors will be reported in the Black GUI area as green reference numbers.

6.13 Apply minimal Loctite 403 glue with fine point nozzle in between LED’s D320 & D316 and D1 & D5. This is to secure D320 and D1, due too small pads easily snapping off during shipping & handling.

# FAILED BOARDS:

## Failed UUTs are to be retested, and if UUT fails a second time, technician will troubleshoot for failures and rework as applicable.

## All reworked UUTs are to be retested.

# TROUBLESHOOTING

8.1 Failed LED’s may just need a small nudge to align them closer to silkscreen.